Attachment 1 to Standard Contract

The United Illuminating Company Meter Requirements

Regarding PA 11-80 – LRECs and ZRECs

- Introduction

This document outlines The United Illuminating Company’s (“UI”) meter requirements for generators participating in the LREC and ZREC program (“Program”) and interconnecting with UI.  This document represents general meter requirements that shall apply to every Program participant.  This document does not replace or render void other UI requirements with respect to any other program or rate tariff applicable to a Program participant (e.g., the requirements for UI’s Net Energy Rider).

If a Program participant already has metering equipment that is capable of communicating the required data to the UI data collection system for use in the Program and such metering equipment meets or exceeds the requirements outlined herein, then the Program participant may submit a written request for UI to review the existing metering equipment at the time that the Program participant submits its application for the Program.  UI may approve or reject use of the existing metering equipment at its sole discretion.

- Programmatic Requirements

  - Meters and meter equipment

All Program participants are required to purchase and install, at their sole cost and expense, meter provisions that, at a minimum, meet the UI Guidebook of Requirements for Electric Service ( http://goo.gl/r7o6c ) as well as the Guidelines for Interconnection (http://goo.gl/5RMwy ).  All meter installations shall meet the accessibility and location requirements listed in the interconnection documents.  Program participants are also solely responsible for any and all costs of all meter provisions and cabinets required as well as the installation of the same.  Program participants are also responsible for expenses (if any) associated with meter installation above and beyond those covered by applicable rates and tariffs.

If UI determines that the meter installation requires instrument transformers, the Program participant will provide, at its sole cost and expense, an approved instrument transformer cabinet for UI’s use.  UI will normally purchase, own and install any required instrument transformers.   In special circumstances, UI may, at its sole discretion, approve the use of instrument transformers provided with the applicable generating equipment (provided that, among other things, the devices supplied with the generating equipment are of revenue grade .3% (.003) accuracy).

Location of meter provisions and cabinets must be in a safe and adequate meter working space as determined by UI in its sole discretion.  Generally, the generator meter provision should be located as close as reasonably possible to the Program participant’s revenue meter(s).  UI will purchase, own, install, maintain, test and read the meters, but the Program participant is responsible for any and all monthly metering fees as established by rates or tariffs approved and on file with the Connecticut Public Utilities Regulatory Authority.  Program participants may install separate energy metering equipment at their sole cost and expense, but this equipment must be completely independent of any equipment used by UI.  UI will not utilize, read, maintain or test any meters or equipment provided with the generating equipment or provided by the participant.

The number of required meters will vary depending on the size of the installation compared to energy consumed by the Program participant and the rates and tariffs applicable to the participant.  Typically (but not always), a standard LREC/ZREC installation will require two meters: a revenue (current Program participant meter) and a generator meter.  The generator meter will be located after any inverters from the generator and before the participant’s load.  Additional metering may be required and will be at the Program participant’s sole cost and expense, such as if the Program participant desires to participate in the UI Net Energy Rider (as detailed in the corresponding tariff).

If power storage equipment (i.e., battery bank) is utilized by the Program participant, the connection for the power storage equipment will be behind the inverter.  Generation will be measured net with the power consumed by the power storage equipment.

  - Application Review

Accompanying the Interconnection Application, UI requires, at a minimum, a bill of materials (including the meter of record), cut sheets, a site plan, and a one-line drawing of the applicable system as well as any other systems located at the same site.  At UI’s request, the Program participant may be required to submit additional information including, but not limited to, three-line drawings, system wide drawings for larger commercial participants as well as specifications of existing equipment within a Program participant’s system.

UI will conduct a review of the Program participant’s system as it affects metering and will have the authority and sole discretion to require changes to any portion of the system as well as to reject any application based upon UI’s judgment.  Additionally, UI will review any other systems located at the same site and will have authority to require changes to existing generation systems on site prior to approving the current Program participant application.  Multiple generation meters on one site will require UI’s review and approval.

Interconnection applications will follow UI’s standard Distributed Generation interconnection process approved and on file at PURA.  UI reserves the right to review and reject any application based upon existing or anticipated system conditions within the proximity of the Program participant’s site or any concerns with respect to system reliability associated with the Program participant’s design.

- Meter Data

UI will provide monthly revenue meter reads to the Program participant as part of the existing rate or tariff pursuant to which Program participant is taking electric service.  UI will provide to NEPOOL GIS the monthly meter reads for the generator meter(s).  Additional information including, but not limited to, daily reads and reactive or interval data, may be available if collected by the revenue meter.  Such additional data can be provided at Program participant’s sole cost and expense pursuant to applicable rates and tariffs.